Orbit Linux Library Manual

Table of Contents

1.	Documentation Cross-Reference	4

2.	Introduction	4

3.	Terminology and Abbreviations	5

4.	Compatibility	6

5.	Included Software	6

6.	Installation	6

6.1	Installation on Ubuntu/Linux Mint 20 LTS using apt	7

6.2	Installation using the tarball.	7

7.	Configuration	8

7.1	RS232 Module Configuration	9

7.2	ETHIM Module Configuration	9

7.3	USBIM Module Configuration	10

7.4	WIM Module Configuration	11

8.	Simple Code Example	11

9.	Including the library in your application.	12

9.1	Orbit Server Class	12

9.2	Orbit Enumerations	13

10.	Connecting to an Orbit Network	13

10.1	OrbitNetworks Class	14

10.2	OrbitNetwork Class Connection Properties	15

10.3	Network Speed	15

10.4	OrbitNetwork Class Network Speed Properties	16

11.	Adding Modules to an Orbit Network	16

11.1	Adding Modules by Orbit Identity	16

11.2	Adding Modules Using the Notify Mechanism	17

11.3	Adding Modules Using Ping	17

11.4	Adding Modules Using Hot Swap	17

11.5	Loading and Saving an Orbit XML File.	18

11.6	Accessing Connected Modules.	18

11.7	Deleting Modules	19

11.8	Clearing Modules	19

11.9	Remapping Modules	19

12.	Module Identity Properties	20

13.	Measurement Modes	20

13.1	Basic Individual Module Readings	21

13.1.1	Readings in Units of Measure	21

13.1.2	Readings in Counts	22

13.2	Readburst – Synchronised Fast Readings.	22

13.3	Difference mode – Synchronised Fast Readings.	23

13.4	Buffered Mode	25

13.4.1	Buffered – Sync Mode	27

13.4.2	Buffered – Sample Mode.	28

13.4.3	Buffered – Sample Mode with External Master.	29

13.5	Dynamic Mode	30

13.5.1	Overview	30

13.5.2	Dynamic Mode Reference	31

13.5.3	Dynamic Example 1	34

13.5.4	Dynamic Example 2	35

14.	Module Status	37

15.	Module Specific Properties	37

15.1	Overview	38

15.2	OrbitModuleAGM (Air Gauge Modules)	38

15.3	OrbitModuleAIM (Analog Input Modules)	40

15.4	OrbitModuleDIOMBase (DIOM and DIOM2 base class)	40

15.5	OrbitModuleDIOM (Digital IO Module)	41

15.6	OrbitModuleDIOM2 (Digital IO Module 2)	41

15.7	OrbitModuleDIM (Digimatic Input Module)	42

15.8	OrbitModuleDP (Digital Probes)	43

15.9	OrbitModuleEIM (Encoder Input Module)	44

15.1	OrbitModuleLE (Linear Encoder)	46

15.2	OrbitModuleLT (Laser Triangulation)	48

15.1	OrbitModuleLTA (Laser Triangulation)	48

15.2	OrbitModuleLTH (Laser Triangulation High Sensitivity)	49

15.3	OrbitModuleWCM (Wireless Communication Module)	50

16.	Wireless Hand Tools	51

16.1	Wireless Handtool Configuration	53

16.1.1	WCM_DeviceSettingsBase	54

16.1.2	WCM_DeviceSettings_WHT	54

16.1.3	WCM_DeviceSettings_WHTM	57

16.1.4	WHT Settings Example	58

17.	Revision History	59

# Documentation Cross-Reference

| 502914 | Orbit3 Modules Manual | Details on installation and electrical requirements. |
| --- | --- | --- |
| 502990 | Orbit3 System Manual | Details on installation and electrical requirements for the Orbit Library compatible products |
| 502989 | Orbit3 Software Manual | Details Orbit Software and the Windows library. |
| 503158 | Orbit LT and LTH Laser Sensor Manual | Details Requirements for Laser products. |

# Introduction

This manual provides instructions for the installation and use of the Orbit Linux Library. It should be read in conjunction with the Orbit Software Manual (502989).

The Orbit Linux Library is a C++ library for communicating with a Solartron Orbit Network on the Linux platform. It has been designed to enable you to easily create applications that use Orbit Network devices on the Linux operating system.

The library supports USBIM, RS232IM, ETHIM and WIM controllers. ETHIM and RS232 controllers must be configured by a text file. A utility OrbitRegistration is provided to update the text file. See the section Configuration for the file format and instructions.

The library allows the configuration of most Orbit Module’s parameters through classes defined for each Module type (for example the resolution and averaging properties of DP modules).

The release tarball includes a script to copy the library, applications, headers and example source code into the standard locations in the Linux filesystem. See the section Installation for full instructions. The source code for the example application OrbitLinuxLibraryTest is located in /usr/src/OrbitLinuxLibraryTest after installation.

The release .deb package (for Ubuntu 20 / Linux Mint 20) will also install files in the same locations as the tarball and will additionally download and install any missing dependencies (provided they are available in the distribution repository) when installed using apt.

Strings returned by the library are of type std::string. Strings of type char* can be accessed using the c_str() method of that class.

# Terminology and Abbreviations

| AGM | Air Gauge Module |
| --- | --- |
| AIM | Analog Input Module |
| APT | Advanced Package Tool. Linux software to install software packages on Debian/Ubuntu based Linux distributions. |
| Bluetooth | A wireless communications standard. |
| D2XX | USB Direct driver. |
| DHCP | Dynamic Host Configuration Protocol. Configures hosts on IP networks. |
| DIM | DigiMatic Input Module |
| DIOM | Digital Input Output Module |
| DIOM2 | Digital input Output Module v2 |
| DP | Digital Probe |
| EIM | Encoder Input Module |
| ETHIM | Orbit Controller to interface between an Orbit Network and an Ethernet network using the TCP/IP protocol (IPv4 only). |
| FTDI | Future Technology Devices International. Semiconductor company. |
| GCC | GNU Compiler Collection. An open source compiler. |
| GNU | Acronym “GNU’s not unix!” A collection of open source software including compilers and operating system kernels. |
| IP | Internet Protocol |
| Kernel | The software that forms the core of an operating system. |
| Linux | An open source Unix like operating system. |
| LE | Linear Encoder |
| LEIM | Linear Encoder Input Module |
| LT | Laser Triangulation |
| LTH | Laser Triangulation High Sensitivity |
| Orbit Controller | Orbit Module to interface between a PC/PLC and an Orbit network |
| Orbit Module | A module (usually DIN mount) that connects to an Orbit network. |
| RS232IM | Orbit Controller to interface between an Orbit network and RS232 |
| STL | Standard Template Library. A C++ library. |
| SUDO | Super User Do. Runs programs as another user, typically the user “root”. |
| TAR | Unix file archive standard (originates from Tape Archive). Often compressed using the gzip utility. |
| TCP | Transmission Control Protocol. A connection-based network protocol. |
| WCM | Wireless Communication Module |
| WHT | Wireless Hand Tool |
| WHT-M | Multichannel Wireless Hand Tool |
| USBIM | Orbit Controller to interface between an Orbit network and USB |
| UDP | User Datagram Protocol. A packet-based network protocol. |
| USB | Universal Serial Bus. A computer interface standard. |
| UDEV | Userspace dev. Creates device nodes and inserts kernel modules on the fly when new hardware is detected. |
| VCP | Virtual COM Port. Created when a USB device operates like a serial port. |
| XML | eXtensible Markup Language. A standard way of formatting files. |

# Compatibility

GNU Linux, x86_64 and i386 processor instruction sets. GCC version 7.2 or above is recommended.

# Included Software

| libOrbitLibrary.so | A shared object DLL your own software can link to. Communicates with Orbit networks. |
| --- | --- |
| OrbitRead | A console application configured with command line arguments to read Orbit Modules and WHT channels. Outputs CSV to stdout. |
| OrbitReadburst | A console application configured with command line arguments to Readburst Orbit networks. Outputs CSV to stdout. |
| OrbitReadDifference | A console utility to perform an Orbit Difference operation. Useful for runout measurements. |
| OrbitReadDynamic | A console application configured with command line arguments to perform Orbit Dynamic 2 collections. Outputs CSV to stdout. |
| OrbitRegistration | A GUI application to configure RS282IMs and ETHIMs |
| OrbitLinuxLibraryTest | An example application to demonstrate library usage (source provided). The utility can also be used to configure WCM modules, WHT handsets and AGM modules. |

# Installation

The library is released as a .deb package and a tarball.

The .deb package is a simpler installation as the package manage will attempt to find and download runtime dependencies, however those dependencies must be available in your distribution’s repository.

The tarball will install the software on any Linux system, however dependencies are not automatically installed and must be downloaded manually.

## Installation on Ubuntu/Linux Mint 20 LTS using apt

- Run apt update to update the list of available packages:

sudo apt update

- Install the package

sudo apt install ./orbitlibray_1.0_amd64.deb

- Answer yes when prompted to continue installation.

- A udev rule will have been installed to enable direct communication with USBIMs (the udev rule is recommended to enable autodetection and the high speed read modes of USBIM modules). See the section USBIM Module Configuration for more details.

We recommend keeping this rule file, however if it is not required it can be removed using the command below. Once removed the device will only be able to communicate with USBIMs in virtual com port mode and USBIMs must be manually configured using OrbitRegistration.

sudo rm /etc/udev/rules.d/51-Solartron.rules

## Installation using the tarball.

- Obtain the correct archive for your operating system architecture (32bit or 64 bit).

- Decompress and extract the archive (replace N.N with the version number):

tar -zxvf libOrbitLibrary.so.N.N

- Change to the archive directory:

cd libOrbitLibrary.so.N.N

- Make the installation script executable

chmod ugo+x install.sh

- Run the supplied installation script. The script will copy the executable, libraries and resources into the appropriate Linux folders.

sudo ./install.sh

- Optionally install the supplied udev rule when prompted to enable direct communication with USBIMs (the udev rule is recommended to enable autodetection and the high speed read modes of USBIM modules). See the section USBIM Module Configuration for more details.

- Use your distribution’s package manager to install any required libraries, if needed. For example: sudo apt-get install libxml2 libicu60 libqt5widgets5

# Configuration

RS232IMs, ETHIMS and WIMs are configured using the OrbitRegistration utility. Settings for the library are saved in the file:

~/.SolartronMetrology/orbitlibrarysettings.xml

This is an XML file containing the configuration for RS232, ETHIM and WIM modules. As the file is located in the user’s home directory, each user has their own configuration.

If a global configuration is preferred, the settings file may be copied into the /etc directory and made readable to all users:

cp ~/.SolartronMetrology/orbitlibrarysettings.xml /etc

chmod o+r /etc/ orbitlibrarysettings.xml

It is possible to write your own configuration file instead of using our utility, an example file that configures and RS232IM, ETHIM and WIM is below.

- For an RS232IM the <comport> and <baud> elements are required and the controllertype attribute must be RS232IM

- For an ETHIM the <address> element is required and the controllertype attribute must be ETHIM.

- For a WIM the <address> and <channel> elements are required and the controllertype attribute must be WIM. Channel is the Bluetooth RFCOMM channel and will be either 1 or 5 depending on the WIM. The OrbitRegistration or Linux sdptool utility can find the RFCOMM channel.

<?xml version="1.0" encoding="utf-8"?>

<settings>

<networks>

<network controllertype="RS232IM">

<networkname>Network 1</networkname>

<comport>/dev/ttyS0</comport>

<baud>9600</baud>

</network>

<network controllertype="ETHIM">

<networkname>Network 2</networkname>

<address>192.168.1.100</address>

<mac>00-20-4A-CA-8B-D9</mac>

<moduleid>398AD12R22</moduleid>

</network>

<network controllertype="WIM">

<networkname>Network 3</networkname>

<address>01:02:03:04:05:06</address>

<channel>5</channel>

</network>

</networks>

</settings>

## RS232 Module Configuration

In most Linux distributions user accounts must be a member of the group ‘dialout’ to gain permission to read and write to serial devices.

- A user account can be added to the dialout group with the command:

sudo usermod -a -G dialout username

- Log the user out and back in to apply the change.

- Use the OrbitRegistration program (run from the command line) to register the serial device the USBIM is attached to. Select the RS232IM baud rate from the drop-down list (the default is 9600) and optionally enter a name for the Orbit Network. Click ‘Save All’ to write the configuration to disk.

## ETHIM Module Configuration

Note: You should always consult your network administrator prior to connecting any device to a network.

Run the OrbitRegistration application from the command line. Select the Ethernet tab. In most cases the ETHIM will have been autodetected. Check the box in the ‘Registered’ column and click ‘Save All’. In cases where the ETHIM is not detected, it is possible to manually enter the IP address of the module by clicking on the ‘Connect to IP’ button.

By default, an ETHIM uses DCHP to obtain its IP address. This is also the recommended configuration.

It is also possible to assign it a static IP address. Use great care when configuring an ETHIM with a static IP address not to lose communication with it.  Make sure it is IP address that is not in use or within a range allocated for assignment by a DHCP server to another device on your network. If unsure, use DHCP.

Consult your network administrator to obtain static IP address allocation and the subnet mask and default gateway of your network. Select the ETHIM configurator tab, select the ‘Use the following static configuration’ checkbox and enter the values provided by your network administrator. Click Apply configuration and the IP configuration will be applied to the ETHIM module.

## USBIM Module Configuration

A USBIM can be used by the library by one of two methods, directly using the libftdi direct driver (recommended) or by using the VCP (virtual COM port) driver included in most Linux kernels. The two drivers are mutually exclusive. It is not possible to have the FTDI VCP kernel module loaded and use the direct driver at the same time. The VCP driver is only appropriate for slow reads. For faster reads and to enable more advanced Orbit modes including dynamic, the direct driver is required.

If using the D2XX direct driver, install the udev rule when prompted by the installation script. This will add a rules file to your /etc/udev/rules.d file to detect the USBIM before the udev rules file of your Linux distribution. The rule file automatically detects the USBIM when it is inserted, applies read and write permission to all user to it and executes a script to unbind the ftdi_sio and usbserial kernel modules. The last step is necessary to use the direct driver as the VCP and direct drivers cannot be used at the same time. The library will now be able to autodetect USBIM modules and no further configuration will be necessary.

If using the VCP driver, when prompted by the installation script DO NOT install the udev rule. Most Linux distributions will autodetect the USBIM, will automatically insert the ftdi_sio and usbserial kernel module and create a device for the USBIM, such as /dev/ttyUSB0. Simply use the OrbitRegistration program and select the baud rate 460800. Save the configuration and the module will be detected as a RS232IM. On most Linux distributions user accounts that are to communicate with the Orbit Network should be members of the ‘dialout’ group to gain read and write permission to the serial device. If using the VCP method USBIMs will behave as RS232IMs in this library.

If you have installed the udev rule for the direct driver and no longer require it, or wish to use the VCP driver instead, simply delete it from the folder /etc/udev/rules.d

sudo rm /etc/udev/rules.d/51-Solartron.rules

The file can be reinstalled by running the install script again.

## WIM Module Configuration

First the WIM must be paired with linux. This can be accomplished at the command line using the bluetoothctl utility. The WIM may be secured with a PIN. This PIN is fixed as 61736.

>bluetoothctl		Starts bluetoothctl. This Linux utility can pair Bluetooth devices.

>scan on		Initiates Bluetooth device discovery.

> devices		This will list all devices including the WIM and its Bluetooth address.

> agent on		Enables bluetoothctl to accept a pairing PIN request from the WIM.

> pair [Bluetooth address of WIM]   	Begins the WIM pairing process. Enter the PIN if requested.

>exit			Exits bluetoothctl.

Next start OrbitRegistration. The WIM should be displayed on the WIM tab (it may take a few seconds for OrbitRegistration to discover the WIM). Click the register checkbox next to the WIM and click Save All.

# Simple Code Example

The program below is a simple example program to read the first module on the first network in units of measure, output the reading to the console and exit.

Prepending of the namespaces std and OrbitLibrary has been avoided by the “using namespace” statement to simplify the code.

#include <OrbitLinuxLibrary/OrbitServer.h>

#include <string>

#include <iostream>

using namespace std;

using namespace OrbitLibrary;

int main(int argc, char *argv[])

{

OrbitServer server;

// Find and connect to all networks

OrbitNetworks *networks = server.Networks();

networks->FindNetworks();

networks->ConnectAll();

// Get a pointer to the first network

OrbitNetwork *firstNetwork = networks->Network(0);

// Add modules using the ping method

firstNetwork->Ping();

// Read the first module in units of measure

OrbitModule *module = firstNetwork->GetModule(0);

cout << module->ReadingInUnits() << endl;

// Disconnect cleanly (disposes all network and module objects)

networks->DisconnectAll();

}

# Including the library in your application.

To use the library in your application, include the header file OrbitServer.h and configure your linker to link to libOrbitLibrary.so (located in /usr/lib). Ensure that the path /usr/include is in the includes path of your project. All library classes are within the OrbitLibrary namespace. Create a single instance of the OrbitServer object

An example:

#include <OrbitLinuxLibrary/OrbitServer.h>

using namespace OrbitLibrary;

OrbitServer myServer;

The OrbitServer object provides library information and provides access to the OrbitNetworks object. The OrbitNetworks object provides access to individual Orbit Networks, which in turn provide access to Orbit Network modules.

## Orbit Server Class

Applications using the Linux Orbit Library should create a single instance of the OrbitServer class. This class provides access to the OrbitNetworks class for connecting to Orbit Networks and provides information about the library. The OrbitServer methods for starting and stopping dynamic collections across all networks are detail separately in this manual.

The OrbitServer object is the only object that your code should create and destroy. All other object instances will be created by the library.

| string LibraryPartNumber(); | The Solartron part number of this library. |
| --- | --- |
| string LibraryVersion(); | The build version number of the library |
| string LibraryBuildDate(); | The build date of the library |
| string GetErrorDescription(int ErrorNumber); | Gets the error associated with an error number |
| OrbitNetworks* Networks(); | Returns an object for finding and connecting to Orbit Networks |

## Orbit Enumerations

The library makes use of many enumerations. Helper functions have been defined to convert enumeration values into std::strings and from std::strings to enumeration values. All the available conversion functions are listed in the header file enumsToString.h.

Examples:

To convert an eModuleType to a string:

eModuleType modType = eModuleType_DP;

cout << eModuleType_ToString(modType) << endl;

To convert a std:string to an enumeration member:

eResolution resolution = eResolution_FromString(“eResolution_res14Bit”);

# Connecting to an Orbit Network

Connecting and Disconnecting from an Orbit network differs from the Windows library. This library does not protect your application against multiple applications accessing the library at the same time. Also networks must first be discovered using the FindNetworks() method of the OrbitNetworks object before connection.

After including the library in your project (see Including the library in your application) the next step is to load the list of networks by calling the FindNetworks() method of the OrbitNetworks object.  This reads the configuration file to find RS232IMs, ETHIMs, WIMs and enumerates USBIMs. The OrbitNetworks object can be retrieved by the Networks() method of the OrbitServer object.

OrbitNetworks *networks = server.Networks();

networks->FindNetworks();

Be aware that on calling the FindNetworks() method the existing list of networks and all child objects will become invalid.

The next step is to connect to the Orbit network(s). Either connect to every network found using the ConnectAll method of the networks object or connect to individual network using the connect method of the OrbitNetwork class. The OrbitNetwork object can be retrieved from the Network() method of the OrbitNetworks class:

Either:

networks->ConnectAll();

Or:

networks->Network(0)->Connect();

The IsConnected() method of the OrbitNetwork class returns if a network is connected or not.

cout << “Connected: “;

cout  << (networks->Network(0)->IsConnected() ? “Yes” : “No”) << endl;

Use the DisconnectAll() or the Disconnect() method of individual networks to close the connection to the Orbit controllers when finished with the network making them available for connection by other applications again and releasing the resources used.

networks->DisconnectAll();

## OrbitNetworks Class

| void FindNetworks(); | Clears existing networks and then finds Orbit Networks by reading the configuration file and enumerating USBIMs. |
| --- | --- |
| void ConnectAll(); | Connects to all Orbit Networks |
| void DisconnectAll(); | Disconnects from all Orbit Networks |
| int Count(); | Returns the number of networks found (both connected and disconnected networks are counted). |
| OrbitNetwork* Network(unsigned int Idx); | Returns a pointer to an Orbit network given its index |

## OrbitNetwork Class Connection Properties

The OrbitNetwork class has properties to manage the connection to an Orbit Network controller. Each network type (USBIM, RS232) has its own descendant class, however there are no public module specific methods or properties.

| virtual void Connect(); | Connects to the network |
| --- | --- |
| bool IsConnected(); | Returns true if the Orbit network is connected |
| string ControllerID(); | Returns the Orbit Identity of the controller in a std::string. |
| string Description(); | Returns a text description of the network, for example USBIM (123AA01001) |
| virtual void Disconnect(); | Disconnects from the Orbit network and releases resources. |
| string NetworkName(); | Returns a user settable name for the network. This can be set in the configuration file for USBIMs and RS232IMs or set using setNetworkName(). |
| void setNetworkName(string networkname); | Sets the user settable name for the network (This will not update the name set in the configuration file). |
| eNetworkType NetworkType(); | Returns the type of network (USBIM, RS232IM etc). USBIMs using the VCP mode will be returned as RS232IMs. |

## Network Speed

The Orbit network can run at various speeds. The default is low speed. The Orbit Network speed is the speed of the Orbit network itself. It is not the speed of the connection between the controller and the PC.

If a higher speed is required (for example dynamic mode requires high speed) then the speed can be changed using the setNetSpeed() method of the OrbitNetwork object.

Modules and controllers can vary in their capabilities, therefore the AvailableSpeeds() method is provided to list the speeds that the current combination of networks are modules are capable of. For example, an RS232IM is not capable of UltraHighSpeed.

The SpeedAvailable() method is also available to test if the current combination of modules and controller are capable of the speed passed as the argument.

## OrbitNetwork Class Network Speed Properties

| vector<eNetSpeed> AvailableSpeeds(); | Returns a std:vector<eNetSpeed> of the speeds that the controller and module combination is capable of. |
| --- | --- |
| eNetSpeed NetSpeed(); | Returns the current speed the network is set to. |
| void setNetSpeed(eNetSpeed Speed); | Sets the network speed to the speed passed as the argument. An exception will be thrown if the speed cannot be set. |
| bool SpeedAvailable(eNetSpeed Speed); | Returns true if the controller and module combination is capable of the speed passed in the argument Speed. |

# Adding Modules to an Orbit Network

Once connected to the controller, Orbit modules must be added to the network before they can be read. A number of difference methods are provided to add modules from simply adding them by their Orbit Identity (printed on the label) to using the hot swap functionality to connect modules back to the network at the address previously stored in their T-CON.

The maximum number of modules that the controller supports can be retrieved by calling the MaximumAllowableNumberOfModules method of the OrbitNetwork object. Attempting to add more modules to the network than this limit will fail.

## Adding Modules by Orbit Identity

To add a module using its Orbit Identity (from the label) call the AddModule method of the OrbitNetwork object specifying the Orbit Identity in the ModuleID parameter:

void AddModule(string ModuleID, string ModuleName);

For example:

network->AddModule(“123AB01001”, “Probe at position 1”);

This method will add the module to the end of the modules list. ModuleName is a property that allows you to give the module a text name which can be read back later using the ModuleName method of the OrbitModule class.

## Adding Modules Using the Notify Mechanism

Orbit Notify is a mechanism that adds modules to an Orbit network by detecting movement of a probe, using the notify button on an air gauge module or a change in an input of a DIOM module. Whilst performing a notify the network is busy and cannot be used for other operations.

On calling the BeginNotifying method of the OrbitNetwork class the library spawns a thread to monitor the network for the notification and returns. The notify thread will continue until either the notification is detected or CancelNotifying is called. The Notifying method returns true whilst a notify operation is in progress. An example implementation is included in the OrbitLinuxLibraryTest application.

Unlike the Windows library, a cancel key cannot be specified as this is not appropriate in a Linux environment.

OrbitNetwork OrbitNotify Properties

| void BeginNotifying() | Begins the notify process by spawning a new thread and continually checking the network for a notified module. |
| --- | --- |
| bool Notifying() | Returns true if a notify operation is in progress. |
| void CancelNotifying(); | Cancels a notify operation. |

## Adding Modules Using Ping

Ping automatically identifies modules on the network and adds them to the modules list. A ping operation will take approximately 12 seconds to complete. Older (non Orbit3) modules will not respond to an Orbit Ping and will not be detected.

To perform an OrbitPing call the Ping method of the OrbitNetwork object:

For example:

server.Networks()->Network(0)->Ping()

## Adding Modules Using Hot Swap

Hot swap is an Orbit function that stores an Orbit Module’s address and type in its T-Con. This allows the module to be quickly changed, with the replacement module assuming the Orbit identity and address of the previous module.  The old module must have been previously connected to the Orbit Network using another method to store its address in the T-Con.

To use hot swap, the modules must remain continuous and be of the same type as the previous module. Hot swap will cease adding modules at the first address that does not contain a module or that is a module of a different type (for example has a different stroke).

Modules without a T-Con (including AGM-B’s and older module types) are not hot swap compatible. The HotSwapCapable method of the OrbitModule class will return true if they module is capable of hotswap functionality.

An example FindHotswapped call:

server.Networks()->Network(0)->FindHotswapped()

The information stored in all the T-Cons connect to the network can be cleared with the ClearTCons method of the OrbitNetwork object. This will clear the memory in every T-Con with a connected module. T-Cons without a module cannot be cleared. This can be useful if rebuilding a network from scratch.

server.Networks()->Network(0)->ClearTCons()

## Loading and Saving an Orbit XML File.

The OrbitNetwork class has Load and Save methods to load and save a standard Orbit XML file. This file contains the Orbit network description, Orbit speed and a list of modules on the network complete with module settings (for example the resolution and averaging setting of a DP). This file is in the same format and interchangeable with the version created by the Windows library.

For Example:

std::string filename = “orbit.xml”;

server.Networks()->Network(0)->Save(filename);

server.Networks()->Network(0)->Load(filename);

## Accessing Connected Modules.

The OrbitNetwork class contains methods to access modules. The methods return an OrbitModule* that can be cast into an object of the appropriate type using dynamic_cast<>();

| unsigned int ModuleCount(); | Returns the number of modules on the Network |
| --- | --- |
| OrbitModule* GetModule(unsigned int Idx); | Retrieves a Module on the network using its index on the network |
| OrbitModule* GetModuleByID(string OrbitIdentity); | Retrieves a module on the network by its Orbit Identity |
| int GetModuleIndexByID(string OrbitIdentity); | Gets a Module Index on the network by its Orbit Identity |

As an example, this code will access the first module on the first network. Assuming that the module is a DP it will set the averaging and output the reading in units of measure.

OrbitNetwork *network = Server.Networks()->Network(0);

OrbitModuleDP *dp = dynamic_cast<OrbitModuleDP *>(network->GetModule(0));

If (dp != NULL}

{

dp->setAveraging(eAveraging_average64);

cout << dp->ReadingInUnits() << dp->UnitsOfMeasure() << endl;

}

## Deleting Modules

A module can be deleted from the Orbit Network by one of the methods below. Although the module will remain physically present on the network, it will be removed from the module list. All modules on the network at indexes after the deleted module will be moved down in the module list by one.

// Deletes a module at the index specified

void DeleteModule(int Index);

// Deletes a module by its module ID

void DeleteModule(string ModuleID);

## Clearing Modules

The ClearModules() method of the OrbitNetwork class will remove every module from the network and reset the network. The network speed will not be altered after this call has completed.

## Remapping Modules

Modules can be moved on an Orbit network by assigning them a new address. This can be useful, for example, in dynamic collections where the modules that form the collection must be contiguous and start from index 0 in the modules list.

To move a module assign it and the module at the index it is to be moved to new addresses using the setRemapIndex() method of the OrbitModule Class. Once every module to be moved has a new address call the ApplyRemap method of the OrbitNetwork class.

For example, to swap the module at index 0 and the module at index 3:

OrbitNetwork *network = server.Networks()->Network(0);

network->GetModule(0)->setRemapIndex(3);

network->GetModule(3)->setRemapIndex(0);

network->ApplyRemap();

# Module Identity Properties

The methods in the table below are provided by the OrbitModule object to aid module identification.

| virtual string DeviceType(); | Returns the Orbit device type of the module, for example: 975326-DA08. |
| --- | --- |
| virtual string GetProbeID(); | If the module has a detachable probe returns the Orbit Identity of the probe. Non detachable probe modules will return the module id of the Orbit Module. |
| virtual string ModuleID() | Returns the Orbit identity of the module. |
| virtual string ModuleName(); | Returns the user settable name assigned to the module (by OrbitNetwork->AddModule or OrbitModule->setModuleName) |
| virtual eModuleType ModuleType(); | Returns an enumeration member corresponding to the module type. For example eModuleType_DP |
| virtual void setModuleName(string modulename); | Sets the user settable module name. |
| virtual eGeneration OrbitGeneration(); | Returns the Orbit generation of the module. For modern modules that will be eGeneration_Orbit3. |
| irtual double Stroke(); | Returns the module’s stroke. E.g. 2 for a DP2. |
| virtual string Version(); | Returns a std::string containing the version number of the module’s firmware. |

# Measurement Modes

Orbit offers many different read modes to provide solutions to the many different requirements of gauging systems. Refer to the Orbit Software Manual (502989) for a full list of the advantages and disadvantages of each mode, read speeds, module compatibility, and the specific requirements of each mode.

Wireless Handtools are not included in these module measurement methods. The wireless communication module will always return zero when using these read modes. To read a handtool use one of the read() methods of the WCM_Device class. See the Wireless Handtools section of this manual for instructions and examples.

## Basic Individual Module Readings

Modules can be read individually either in units of measure or counts using the ReadingInUnits() and ReadingInCounts() methods of the OrbitModule class. This provides the reading from a single Orbit module. This is the simplest but slowest method of reading a module and does not provide synchronised readings.

Counts are the raw readings sent over the Orbit Network and are integers utilising the full bit count of the resolution that the module is set to. For example, a module with 14-bit resolution will read 214 (16384) at full stroke and zero at the beginning of the stroke.

Readings in Units are readings provided in the units of measure converted from counts by the library.

If the probe is in an under-range position, then the offset will be returned (or zero if the scale does not have an offset). If the probe is over-range, then full scale (offset + stroke) will be returned. For example, a -1V to 1V AIM will read -1V if under range and 1V if over range.

The OrbitModuleStatus class should be used to check for error conditions including over range, under range and communications timeout. It can be retrieved with the ModuleStatus() method of the OrbitModule class.

### Readings in Units of Measure

Readings in units of measure can be retrieved with the ReadingInUnits() method of the OrbitModule class. In addition a std::string containing a text description of the units of measure can be obtained with the UnitsOfMeasure() method.

| virtual double ReadingInUnits(); | Returns a double containing the module reading. |
| --- | --- |
| virtual string UnitsOfMeasure(); | Returns a string containing the units of measure. |

Example:

OrbitNetwork *network = server.Networks()->Network(0);

OrbitModule *module = network->GetModule(0);

cout << “Reading” << module->ReadingInUnits() << module->UnitsOfMeasure() << endl;

### Readings in Counts

Raw counts readings can be reading using the ReadingInCounts() method of the OrbitModule class. The methods Offset() and CPUOM() are provided which provide the values that are used to convert a counts reading to a reading in units of measure. For example to convert counts to units at 18-bit resolution, use this formula:

Units = (16 * CPUOM() * ReadingInCounts() ) + Offset();

| virtual double CPUOM(); | CPUOM at 14-bit resolution. This must be multiplied by 4 if using a 16-bit resolution and 16 if using an 18 bit resolution. |
| --- | --- |
| virtual double Offset(); | The offset from zero that is the beginning of the stroke. |
| virtual int ReadingInCounts(); | Returns an integer containing the reading in counts. |

Example:

OrbitNetwork *network = server.Networks()->Network(0);

OrbitModule *module = network->GetModule(0);

cout << “Reading In Counts: ” << module->ReadingInCounts() << endl;

## Readburst – Synchronised Fast Readings.

An Orbit Readburst commands a network to provide a set of synchronised readings. It is a quick method of reading every module on a network. Readburst is supported by all module types except older (non Orbit-3) modules and wireless handtool channels.

Readburst is initiated by calling the Readburst() method of the OrbitNetwork class. The readburst readings are stored in properties of the OrbitModule class for each module on the network. Both Counts and Units measurements can be read with readburst.

If there are older, non Readburst capable modules on the network, they must be after the modules to be read in the modules list.

OrbitNetwork Readburst Methods

| eOrbitErrors Readburst(int &NumberOfModules); | Performs an Orbit Readburst, returning the Readburst command status and setting the NumberOfModules parameter to the number of modules Readbursted. |
| --- | --- |
| bool ReadBurstCapable(); | This identifies if the both the controller is read burst capable and there are contiguous modules capable of ReadBurst starting at the first address. |
| int ReadBurstCapableCount(); | Returns the number of contiguous modules that are readburst capable starting at address the first address |

OrbitModule Readburst Methods

| virtual eOrbitErrors LastReadburst_Error(); | Returns the error value of the last successful Readburst for this module. This can be used to check for overrange and underrange. |
| --- | --- |
| virtual int LastReadburst_InCounts(); | Returns the counts reading of the last successful Readburst for this module |
| virtual double LastReadburst_InUnitsOfMeasure(); | Returns the reading of the last successful Readburst for this module in units of measure |
| virtual bool ReadburstCapable(); | Returns true if this module is Readburst capable. |

Example:

OrbitNetwork *network = server.Networks()->Network(0);

int modulesRead;

network->Readburst(modulesRead);

for (int index = 0; index < modulesRead; index++)

{

cout << network->GetModule(index)->LastReadburst_InUnitsOfMeasure() << endl;

}

## Difference mode – Synchronised Fast Readings.

In difference mode, the Orbit Module continually reads the probe and reports the minimum, maximum, sum and number of readings taken. This means that the probe is read at a fast rate (244 times per second) independently by the module and the difference values only are transmitted across the network when requested by the controlling software. This makes it particularly useful for runout calculations.

The NetworkDifference class returned by the Difference() method of the OrbitNetwork class is responsible for starting and stopping a difference operation.

The ModuleDifference class returned by the Difference() method of the OrbitModule class is responsible for commanding a read of the difference values and holding a copy of the values read. Values can be read at any time by calling the ReadDifference() method.

Note:

Difference mode requires that modules be set at 14bit resolution.

Linear encoders do not return the sum and number of readings.

Network Difference Methods

| void Start(); | Starts an OrbitDifference operation on all enabled modules. |
| --- | --- |
| void Stop(); | Stops an in progress OrbitDifference Operation. This will also read the final difference value for all modules. |
| int Count(); | Returns the number of modules enabled for difference mode. |
| eDifferenceModeStatus Status(); | Returns the difference mode status: None, started, stopped or reset. |

Module Difference Methods

| void ReadDifference(); | Reads the difference value from the module. This can be called at any time during a difference operation. This must be called before using any of the value properties, diff, max, min etc. |
| --- | --- |
| int ReadingCount(); | The number of readings that the module has taken. |
| bool Enabled(); | Returns true if the module is enabled for difference mode. False if not. |
| void setEnabled(bool EnableState); | Enables or Disables the module for difference mode. |
| eOrbitErrors Error(); | eOrbitErrors_NoError or the error code if the module encounters an error, including overrange, under range, timeout or any other error. |
| eDifferenceModeStatus Status(); | Returns the difference mode status: None, started, stopped or reset. |
| double Diff(); | The difference between the maximum and minimum reading in units of measure. |
| int DiffCounts(); | The difference between the maximum and minimum reading in counts, |
| double Max(); | The maximum value read in units of measure. |
| int MaxCounts(); | The maximum value read in counts. |
| double Min(); | The minimum value read in units of measure. |
| int MinCounts(); | The minimum value read in counts |
| double Sum(); | Sum of all the readings in units of measure. |
| int64_t SumCounts(); | Sum of all the readings in counts. |

Example:

// Get the difference objects

NetworkDifference *netDiff = network->Difference();

ModuleDifference *modDiff = network->GetModule(0)->Difference();

// Enable the module for difference.

modDiff->setEnabled(true);

// Begin the difference operation

netDiff->Start();

// Print out intermediate values

for(int i=0; i<10; i++)

{

modDiff->ReadDifference();

cout << "Difference: " << modDiff->Diff() << endl;

sleep(1);

}

// Stop the difference operation and print out the list difference value recorded

netDiff->Stop();

cout << "Final Difference Value: " << modDiff->Diff() << endl;

## Buffered Mode

In buffered mode the Orbit module will read the probe and store up to 30,000 readings. This is a fast reading mode and provides synchronised readings. Not all module types are compatible with buffered mode.

Buffered mode has two sub modes, sync and sample. Module can be configured with a mixture of both the modes of operation if required.

- In sync mode the modules will take readings at predetermined time intervals.

- In sample mode the modules will take readings on receiving sample signals either from the controller or an external master.

There are two classes to control buffered in the library. ModuleBuffered configures a module for buffered mode and stores data once it is read back by the library. NetworkBuffered starts and stops buffered and issues sample commands.

To perform a buffered read, follow these steps:

1)	Configure the mode, sync or sample.

2)	If using sync set the time interval.

3)	If using sample and an external master, set the master index and configure the external master.

4)	Enable the module for buffered mode.

5)	Start buffered mode.

6)	Wait for the readings to be taken.

7)	Stop buffered mode. The library will read all the modules at this point.

8)	Retrieve the readings.

9)	Disable the modules enabled for buffered mode.

NetworkBuffered Methods

| void Start(); | Sets all modules into buffered mode and then broadcasts a message to begin taking readings . |
| --- | --- |
| void Stop(); | Reads all module readings and then resets all modules out of buffered mode |
| eBufferedModeStatus Status(); | Returns the current buffered mode status (None, reading or finished). |
| int EnabledCount(); | Returns the number of modules enabled for buffered mode. |

ModuleBuffered Methods

| bool Enabled(); | Returns true if the module is enabled for buffered mode. |
| --- | --- |
| void setEnabled(bool value); | Enables or disables the module for buffered mode. |
| eBufferedMode Mode(); | Returns the modules buffered mode configuration: Not configured, configured for sync (timing based) or configured for sample (module waits for a signal to take readings). |
| void setMode(eBufferedMode mode); | Sets the modules buffered mode configuration: Not configured, configured for sync (timing based) or configured for sample (module waits for a signal to take readings). |
| eBufferedModeStatus Status(); | Returns the buffered mode status for this module: None, reading or finished. |
| sReading GetBufferedItem(int index); | After buffered mode has completed, this method is used to retrieve a buffered data item given the index of the item. |
| int BufferedDataLength(); | Returns the number of data items stored. As modules in buffered mode can have different configurations, this value will vary for each module on the network. |
| sReading* GetBufferedDataArray(int &length); | Returns a malloc’d array of the buffered data received. It is the responsibility of the application that calls this method to free this memory. |

sReading Structure

| eOrbitErrors Error; | The error value for this reading. Will usually be eOrbitErrors_NoError. |
| --- | --- |
| int Readcounts; | The raw reading in counts. |
| double Reading; | The reading converted to the modules units of measure. |

### Buffered – Sync Mode

In sync mode, the module will collect readings at time intervals determined by its own internal clock. The timing interval is specified in OTUs (Orbit Timing Units). Use the setMode() method of the ModuleBuffered class to set the module into sync mode.

The ModuleBuffered class contains the methods setOTUs() and  setReadingInterval().

- setOTUs() sets reading timer in Orbit Timing Units (an OTU is 102.4 microseconds). The minimum value is 40 (4096 microseconds). The value can be read back using the OTU) method.

- setReadingInterval sets the module to the nearest OTU in microseconds. That interval error can be read back using the ReadingIntervalErrorUs method.

ModuleBuffered Sync Mode Methods

| int OTUs(); | Returns the number of OTUs between readings. |
| --- | --- |
| void setOTUs(int value); | Sets the number of OTU (Orbit Timing Units) between readings. |
| int ReadingInterval(); | Returns the interval between readings in microseconds. |
| void setReadingInterval(int value); | Sets the reading interval in microseconds. The actual value will be set in Orbit Timing Units. The difference between this and the milliseconds value passed to this method can be read with ReadingIntervalErrorUs. |
| int ReadingIntervalErrorUs(); | Returns the difference (in microseconds) between the microseconds value set using setReadingInterval and the actual reading rate. |

Example:

// Get the buffered objects

NetworkBuffered *netBuffered = network->Buffered();

ModuleBuffered *modBuffered = network->GetModule(0)->Buffered();

// Enable the module for buffered mode and configure.

modBuffered->setMode(eBufferedMode_Sync);

modBuffered->setReadingInterval(100000); // 100ms

modBuffered->setEnabled(true);

// Begin reading

netBuffered->Start();

// Wait 2 seconds

sleep(2);

// Stop reading

netBuffered->Stop();

// Print out the readings

for(int i=0; i<modBuffered->BufferedDataLength(); i++)

{

cout << "Reading " << i << ": " << modBuffered->GetBufferedItem(i).Reading << endl;

}

// Disable buffered mode for the module

modBuffered->setEnabled(false);

### Buffered – Sample Mode.

In sample mode, the module waits for a sample message before taking a reading. The NetworkBuffered class has a method Sample() to broadcast that message from the controller.

Note:

If the network has previously been set to use an external master, that must be cleared first by calling the NetworkBuffered class setMasterIndex method with the value -1.

NetworkBuffered  Sample Mode Methods

| void Sample(); | Broadcasts a message to all modules in buffered sample mode to store a reading. |
| --- | --- |

Example:

// Get the buffered objects

NetworkBuffered *netBuffered = network->Buffered();

ModuleBuffered *modBuffered = network->GetModule(0)->Buffered();

// Enable the module for buffered mode and configure.

modBuffered->setMode(eBufferedMode_Sample);

modBuffered->setEnabled(true);

// Begin reading

netBuffered->Start();

// Send 5 sample commands

for(int reading=0; reading<5; reading++)

{

netBuffered->Sample();

}

// Stop reading

netBuffered->Stop();

// Print out the readings

for(int i=0; i<modBuffered->BufferedDataLength(); i++)

{

cout << "Reading " << i << ": " << modBuffered->GetBufferedItem(i).Reading << endl;

}

// Disable buffered mode for the module

modBuffered->setEnabled(false);

### Buffered – Sample Mode with External Master.

In sample mode, the sample signals can be generated by an external master. Currently EIM and DIOM2 modules can generate the sample signal. The module to be the external master must be at the end of the network module list

Use the setMasterIndex() method of the NetworkBuffered class to set the index of the buffered master. This same method can be used to set the controller to be the master again by passing it the value -1.

The setTxSample() method of the OrbitModuleEIM and OrbitModuleDIOM2 classes set the number of counts the external master changes by between sending Sample signals.

EIM Modules will start sending sample signals after the ref mark has been passed and will be reset to Ref Action: None after the buffered collection has stopped.

NetworkBuffered External Master Methods

| int MasterIndex(); | Returns the index of the external master in the module list. If the value returned is -1, the controller is master. |
| --- | --- |
| void setMasterIndex(int index); | Sets the module in the module list at index to be the external master. Pass the index -1 to set the controller as master. |

OrbitModuleEIM and OrbitModuleDIOM2 External Master Methods

| int16_t TxSample(); | Counts between sample signals in external master mode. |
| --- | --- |
| void setTxSample(int16_t value); | Sets the counts between sample signals in external master mode. |

Example:

// Get the buffered objects

NetworkBuffered *netBuffered = network->Buffered();

ModuleBuffered *modBuffered = network->GetModule(0)->Buffered();

// Enable the module for buffered mode and configure.

modBuffered->setMode(eBufferedMode_Sample);

modBuffered->setEnabled(true);

// Set the master index (in this case the second module

// on the network is assumed to be an EIM) and set it to

// one sample per 100 counts.

netBuffered->setMasterIndex(1);

OrbitModuleEIM *eim = dynamic_cast<OrbitModuleEIM *>(network->GetModule(1));

eim->setTxSample(100);

// Begin reading

netBuffered->Start();

// wait 5 seconds for readings

sleep(5);

// Stop reading

netBuffered->Stop();

// Print out the readings

for(int i=0; i<modBuffered->BufferedDataLength(); i++)

{

cout << "Reading " << i << ": " << modBuffered->GetBufferedItem(i).Reading << endl;

}

## Dynamic Mode

### Overview

Dynamic is a complex mode with many options and requirements. Refer to the Orbit Software Manual (502989) for the full list of options and requirements to use dynamic.

Dynamic and Dynamic 2 stream readings from the module to the PC. It is a method of collecting fast synchronised readings that enables larges amounts of data to be continually streamed to the PC.

This library creates threads to collect dynamic data as it is streamed allowing applications that capture the data to be easily created.

- Traditional dynamic modes (1,2 and 4K) stream readings at a set rate.

- With Dynamic2 the rate can be set with the DynamicInterval() method.

- It is also possible to use an external master to trigger syncs.

Care must be taken not to exceed the bandwidth of the Orbit Network or data will be lost. Refer to the Orbit Software Manual (502989).

Dynamic modes are only available with a USBIM and with the D2XX (direct) FTDI driver. The network must be set to high speed for traditional dynamic. Dynamic 2 operates at the network speeds high and ultrahigh. All modules that are to be part of the network must be positioned contiguously together at the start of the module list.

Dynamic is controlled using the OrbitDynamic class at the network level. The object can be retrieved from an OrbitNetwork object using the Dynamic() method().

Dynamic can also be controlled across all networks using the methods in the OrbitServer class.

### Dynamic Mode Reference

OrbitServer Dynamic Methods

| void StartAllDynamic() | Starts all prepared dynamic collections across all networks. |
| --- | --- |
| void StartAllDynamic(DynamicAllCompletedCallbackFunc callback); | Starts all prepared dynamic collections across all networks. Calls the callback function once all dynamic collections have completed. |
| void StopAllDynamic(); | Stops all collections on all networks. |
| bool DynamicInProgress(); | Returns true if a dynamic collecton is in progress on any network. |

OrbitDynamic Methods

| int CollectionSize(); | Returns the configured collection size (readings from each module). |
| --- | --- |
| void setCollectionSize(int size); | Sets the collection size (readings from each module). |
| OrbitDynamicData* DynamicData(); | Returns an object that stores and manages the dynamic data collected. |
| int DynamicInterval(); | Returns the configured interval between readings in microseconds. Only valid in Dynamic2Custom Rate. |
| void setDynamicInterval(int value); | Sets the interval between readings in microseconds. The dynamic rate must be set to Dynamic2Custom |
| eDynamicMode DynamicMode(); | Returns if dynamic is configured to use an internal or external master to generate syncs |
| void setDynamicMode(eDynamicMode mode); | Sets the dynamic collection to use an internal or external master. If using an external master set its index with setMasterIndex(int); |
| eDynamicRate DynamicRate(); | Returns the configured dynamic rate (1K, 2K, 4K or dynamic2custom) |
| void setDynamicRate(eDynamicRate rate); | Sets the configured dynamic rate (1K, 2K, 4K or dynamic2custom). If using dynamic 2 the interval must also be set using setDynamicInterval(int) |
| eDynamicState DynamicState(); | Returns the current dynamic state (inactive, prepared, running, completed etc.) |
| bool Enabled(); | Returns true if the network has been enabled for dynamic collection. |
| void setEnabled(bool enablenetwork); | Sets the network enabled for dynamic collection or clears it. Enabling the network validates all the settings. |
| int MinimumInterval(); | This returns the minimum value, in microseconds that the dynamic mode can be set to with the current network speed and combination of modules set for the dynamic collection. |
| int MasterIndex(); | Returns the index in the module list of the master module if using an external master. |
| void setMasterIndex(int index); | Sets the index of the module to be master if using an external master. The Dynamic mode must also be set to eDynamicMode_External. |
| void Prepare(); | Prepares dynamic mode to start. This ensures that networks start quickly when StartDynamic() is called. |
| int NumberOfModules(); | Returns the number of modules configured to collect data from. |
| void setNumberOfModules(int count); | Sets the number of modules to collect data from. The modules must be contiguous from the start of the modules list. Difference dynamic rates support different numbers of modules: 1K: 8 modules max. 2K 16 modules max. 4K 31 modules max. Dynamic2Custom: All modules on the network. |
| void StartDynamic(); | Begins a dynamic collection on this network. |
| void StartDynamic(DynamicCompleteCallbackFunc callback); | Starts a dynamic collection on this network. Calls the callback function once all dynamic collection has completed. |
| void StopDynamic(); | Stops a dynamic collection in progress. This is useful to stop a dynamic collection set to continuous or to abort a collection in progress. |

OrbitDynamicData Methods

| int ModuleCount(); | Returns the number of modules (columns) in the dynamic collection. |
| --- | --- |
| int ReadingCount(); | Returns the number of readings (rows) in the dynamic collection. |
| sReading GetItem(int ModuleIndex, int ReadingIndex); | Gets a single reading in units of measure in the dynamic collection. |
| eOrbitErrors GetReadingErrorStatus(int ModuleIndex, int ReadingIndex); | Gets any reading error of a reading in the dynamic collection. This is useful to detect overrange and under-range errors. |
| int GetReadingInCounts(int ModuleIndex, int ReadingIndex); | Returns a single raw counts reading in the dynamic collection. |
| OrbitErrors CollectionStatus(); | Contains any errors encountered collecting dynamic data from the modules. |

OrbitModuleEIM Dynamic Methods

| int16_t TxSync(); | Returns the number of counts configured between sync pulses when this module is configured as a dynamic master. |
| --- | --- |
| void setTxSync(int16_t value); | Sets the counts value between sync pulses when this module is configured as a dynamic master. |
| int16_t HoldOff(); | Returns the number of counts configured to wait before issuing the first sync pulse when this module is a dynamic master. |
| void setHoldOff(int16_t value); | Sets the number of counts to wait before issuing the first sync pulse when this module is a dynamic master. Dynamic master mode must also be set to holdoff when using this property. |
| eDynamicMasterMode DynamicMasterMode(); | Returns the action to take on starting dynamic when configured as a dynamic master. |
| void setDynamicMasterMode(eDynamicMasterMode value); | Sets the action to take on starting dynamic master. Options are: Instant: Start issuing syncs right away. Holdoff: Wait a number of counts before issuing syncs. The value to wait is set using setHoldOff(). RefAction_Reset: Start triggering syncs at the refmark, setting the module reading to zero counts. Refaction_Preset: : Start triggering syncs at the refmark, setting the module reading to a specific number of counts. The value is set using setPresetInCounts(); |
| int PresetInCounts(); | Get the configured preset value for this module |
| void setPresetInCounts(int preset); | This sets the value used to preset the module. If RefAction() is set to None, the module will be immediately preset. If RefAction() is ContinuousPreset or Preset() this is the value preset on passing the refmark. |

OrbitModuleDIOM2 Dynamic Methods

| int16_t TxSync(); | Returns the number of counts configured between sync pulses when this module is configured as a dynamic master. |
| --- | --- |
| void setTxSync(int16_t value); | Sets the counts value between sync pulses when this module is configured as a dynamic master. |
| eDynamicMasterMode DynamicMasterMode(); | Returns the action to take on starting dynamic when configured as a dynamic master. |
| void setDynamicMasterMode(eDynamicMasterMode value); | Sets the action to take on starting dynamic master. The only option for this module type is: Instant: Start issuing syncs right away. |

sReading Structure

| eOrbitErrors Error; | The error value for this reading. Will usually be eOrbitErrors_NoError. |
| --- | --- |
| int Readcounts; | The raw reading in counts. |
| double Reading; | The reading converted to the modules units of measure. |

### Dynamic Example 1

This example demonstrates how to use Dynamic 2 to collect 100 reading from a single DP with an interval of 50ms between readings.

// Add our DP module to the network

network->AddModule("504AL03P01", "DP1");

// Set the network speed to High

network->setNetSpeed(eNetSpeed_High);

// Get the dynamic data object

OrbitDynamic *dynamic = network->Dynamic();

// Setup dynamic

dynamic->setDynamicMode(eDynamicMode_Normal); 		// Syncs generated by the controller

dynamic->setDynamicRate(eDynamicRate_Dynamic2Custom);	// Dynamic 2 (custom read rate)

dynamic->setDynamicInterval(50000);				// Set the reading interval to 50ms

dynamic->setCollectionSize(100);				// Collect 100 readings

dynamic->setNumberOfModules(1);				// Collect from a single module

// Enable dynamic mode, this validates the settings

dynamic->setEnabled(true);

// Prepare, this sets up the library to start reading quickly.

// Useful if multiple networks are to be started at the same time.

dynamic->Prepare();

// Start the actual dynamic collection. You may wish to pass a callback function here.

dynamic->StartDynamic();

// Wait for the dynamic collection to complete. We just loop for simplicity.

while(dynamic->DynamicState() == eDynamicState_Reading || dynamic->DynamicState() == eDynamicState_Processing)

{

usleep(1000); // Sleep 1ms

// You can also stop the dynamic collection using the StopDynamic method.

// dynamic->StopDynamic();

}

// Print out the data collected

OrbitDynamicData *data = dynamic->DynamicData();

for(int reading=0; reading < data->ReadingCount(); reading++)

{

cout << "Reading " << reading << ": " << data->GetItem(0, reading).Reading << endl;

}

### Dynamic Example 2

This example demonstrates how to use Dynamic2 to collect 100 readings from a network consisting of 3 DP modules using an EIM as an external master to generate syncs.

// Add our DP modules to the network

network->AddModule("504AL03P01", "DP1");

network->AddModule("504AL03P02", "DP2");

network->AddModule("504BC24P03", "DP3");

// Add the EIM at the end of the network

network->AddModule("068AM27502", "EIM");

// Set the network speed to High

network->setNetSpeed(eNetSpeed_High);

// Get the dynamic data object

OrbitDynamic *dynamic = network->Dynamic();

// Setup dynamic

dynamic->setDynamicMode(eDynamicMode_External); 		// External Master

dynamic->setMasterIndex(3);					// Our master is the 4th module.

dynamic->setDynamicRate(eDynamicRate_Dynamic2Custom);	// Dynamic 2

dynamic->setCollectionSize(100);				// Collect 100 readings

dynamic->setNumberOfModules(3);				// Collect from 3 modules

// Setup our dynamic external master

OrbitModuleEIM *eim = dynamic_cast<OrbitModuleEIM *>(network->GetModule(3));

eim->setHoldOff(0);		// Start sending syncs immediately

eim->setTxSync(50);		// Generate a sync after every 50 counts

// Set the encoder ref action to no action

eim->setDynamicMasterMode(eDynamicMasterMode_Instant);

// Enable dynamic mode, this validates the settings

dynamic->setEnabled(true);

// Prepare, this sets up the library to start reading quickly.

// Useful if multiple networks are to be started at the same time.

dynamic->Prepare();

// Start the actual dynamic collection. You may wish to pass a callback function here.

dynamic->StartDynamic();

// Wait for the dynamic collection to complete. We just loop for simplicity.

while(dynamic->DynamicState() == eDynamicState_Reading || dynamic->DynamicState() == eDynamicState_Processing)

{

usleep(1000); // Sleep 1ms

// You can also stop the dynamic collection using the StopDynamic method.

// dynamic->StopDynamic();

}

// Print out the data collected

OrbitDynamicData *data = dynamic->DynamicData();

for(int reading=0; reading < data->ReadingCount(); reading++)

{

cout << "Reading " << reading << ": ";

for (int moduleidx = 0; moduleidx < data->ModuleCount(); moduleidx++)

{

cout << moduleidx << "=" << data->GetItem(moduleidx, reading).Reading << "   ";

}

cout << endl;

}

# Module Status

The module status object contains the module status value and any error state that the module is in.

The module status object will only contain valid values if the module is still communicating on the Orbit network. Otherwise the error code will be timeout.

The ErrorCode will be updated and should be checked after performing basic orbit reads to check for under-range and overrange conditions.

The object can also be updated at any time by calling UpdateStatus();

OrbitModuleStatus Reference

| int Status(); | Returns the module status value from the last update, for example is a new reading ready. It is not usually necessary to use this value in your applications. |
| --- | --- |
| eOrbitErrors ErrorCode(); | Returns the ErrorCode from the last module ReadingInCounts(), ReadingInUnits() or the last time UpdateStatus() was called. |
| string ErrorString(); | Returns a std::string description of the error. |
| void UpdateStatus(); | Queries the module and updates the status and error values. |

# Module Specific Properties

## Overview

Orbit modules have properties to configure the measurements taken depending on the module capabilities. C++ Inheritance is used as many module types share common functionality. A UML class diagram is provided to show an overview of the class structure of the library.

## OrbitModuleAGM (Air Gauge Modules)

Inheritance Chain:

OrbitModuleAGMOrbitModuleDPOrbitModule

Air gauge modules must be mastered before use. AGM-A’s have a screen and can be mastered directly on the module. AGM-A’s and AGM-B’s can be mastered via this library.

An example mastering implementation is included in the OrbitLinuxLibraryTest application (agmmaster.h, agmmaster.cpp and agmmaster.ui).  You may use this dialog in your applications. Mastering properties are only applied to readings after the ApplyMastering() method has been called.

Once mastered an AGM operates as a DP.

If an AGM is mastered using the AGM screen it will raise an exception on the next read as the reading parameters may have changed (stroke etc.)  To clear this error and enable readings call the UpdateInfo() method to re-read the module info and prepare the module for readings again.

OrbitModuleAGM has methods both to master the AGM and to configure other properties of AGM modules. Some OrbitModuleAGM properties are only applicable to AGM modules with screens.

OrbitModuleAGM Mastering Methods

| bool IsMastered(); | Returns true if the module has been mastered. |
| --- | --- |
| double MasterMinReadingInUOM(); | Returns the first master dimensions in mm (e.g. 35.1mm). |
| void setMasterMinReadingInUOM(double value); | Sets the first master dimensions in mm (e.g. 35.1mm). |
| double MasterMaxReadingInUOM(); | Returns the second master dimensions in mm (e.g. 35.5mm). |
| void setMasterMaxReadingInUOM(double value); | Sets the second master dimensions in mm (e.g. 35.5mm). |
| double MasterMinPressure() | Returns the first master pressure in psi (e.g. 23.03psi). |
| void setMasterMinPressure(double value); | Sets the first master pressure in psi (e.g. 23.03psi). |
| double MasterMaxPressure() | Returns the second master pressure in psi (e.g. 25.88psi). |
| void setMasterMaxPressure(double value); | Sets the second master pressure in psi (e.g. 25.88psi). |
| double PressureReading(); | Returns the current pressure sensor reading in mm. |
| void ApplyMastering(); | Applies the mastering values set previously. |
| void UpdateInfo(); | Re-reads the module info to update the stroke and other parameters. This should be called after the module is mastered on screen to clear the invalid info error. |

Simplified Mastering Example:

// First collect the mastering parameters by instructing the

// user to insert the masters and reading the PressureReading() method

double minReading = 35.1;

double minPressure = 23.03;

double maxReading = 35.5;

double maxPressure = 25.88;

// Assuming at that the agm is the first module on the network

OrbitModuleAGM *agm = dynamic_cast<OrbitModuleAGM *>(network->getModule(0));

agm->setMasterMinPressure(minPressure);

agm->setMasterMaxPressure(maxPressure);

agm->setMasterMinReadingInUOM(minReading);

agm->setMasterMaxReadingInUOM(maxReading);

agm->ApplyMastering();

OrbitModuleAGM Other Methods

| eAGM_UOM DisplayUnitsOfMeasure(); | Returns the UOM shown on the AGM display. Orbit readings are always in mm. |
| --- | --- |
| void setDisplayUnitsOfMeasure(eAGM_UOM UOM); | Sets the UOM on the AGM display. |
| eAGM_MeasurementMode MeasurementMode(); | Returns the AGM measurement mode, zero to stroke or absolute. |
| void setMeasurementMode(eAGM_MeasurementMode mode); | Sets the AGM measurement mode, zero to stroke or absolute. |
| bool OutputReadingAsPressure(); | Returns if the AGM is set to output a 14-bit count reading representing 0-30psi or normal readings. |
| void setOutputReadingAsPressure(bool value); | Sets the AGM is set to output a 14-bit count reading representing 0-30psi (true) or normal readings (false). |
| int Passcode(); | Returns the AGM passcode restricting access to the menu screens. |
| void setPasscode(int value); | Sets the AGM passcode restricting access to the menu screens. It must be an integer in the range 1 to 99999999. Set to 0 for no passcode. |
| eAGM_RotateDisplay RotateDisplay(); | Returns the AGM display orientation. |
| void setRotateDisplay(eAGM_RotateDisplay rotation); | Sets the AGM display orientation. |
| bool RotateKeyboard(); | Returns true if the keyboard rotation is set to follows the screen rotation. |
| void setRotateKeyboard(bool rotate); | Sets the AGM keyboard rotation. Set true for the rotation to follow the screen rotation. |
| bool ShowPSIReading(); | Returns true if PSI readings are shown on screen. |
| void setShowPSIReading(bool showPSI); | Enables or disables the pressure reading display on the AGM |
| double LimitHigh(); | Returns the AGM high limit in the units of measure that the display is set to. |
| void setLimitHigh(double value); | Sets the AGM high limit in the units of measure that the display is set to. |
| double LimitLow(); | Returns the AGM low limit in the units of measure that the display is set to. |
| void setLimitLow(double value); | Sets the AGM low limit in the units of measure that the display is set to. |
| int EndBandSizeMicrons(); | Returns the AGM end-band size in whole microns. This is the region of the reading Stroke above and below the masters. |
| void ResetFactorySettings(); | Resets the AGM to factory default settings. |
| bool HasDisplay(); | Returns true if this is an AGM model with a display. |

## OrbitModuleAIM (Analog Input Modules)

Inheritance Chain:

OrbitModuleAIMOrbitModuleDPOrbitModule

Analog input modules inherit from DP modules. There are no AIM specific properties or methods, AIMs act like DPs.

## OrbitModuleDIOMBase (DIOM and DIOM2 base class)

Inheritance Chain:

OrbitModuleDIOMBaseOrbitModule

Orbit DIOM and DIOM2 modules provide digital inputs and outputs. The OrbitModuleDIOMBase contains methods shared between both DIOM and DIOM2 modules. DIOM and DIOM2 inherit from this class where the functionality is the same for both module types.

Note: The list of pins is zero based.

DIOM: Pins can be inputs or outputs.  Pins are numbered 0 to 7.

DIOM2: Pins are fixed inputs or outputs:

Outputs: pins are indexed 0 to 3.

Inputs: pins are indexed 4 to 9.

OrbitModuleDIOMBase Methods

| virtual eDiomIoPinConfig PinConfig(int PinNumber) = 0; | Returns the Pin Configuration for the given pin number (input or output). Note: DIOM2 modules have fixed inputs and outputs. |
| --- | --- |
| virtual eDiomIoPinState InputPinState(int PinNumber) = 0; | Returns the pin state for an input pin (high or low). |
| virtual eDiomIoPinState OutputPinState(int PinNumber) = 0; | Returns the pin state for an output pin (high or low). |
| virtual void setOutputPinState(int PinNumber, eDiomIoPinState PinState) = 0; | Sets an output pin state (high or low). |
| virtual int PinCount() = 0; | Returns the number of pins. DIOMs return 8, DIOM2s return 10. |
| virtual eDiomInputDebounceTime DebounceTime() = 0; | Returns the current debounce time setting. |
| virtual void setDebounceTime(eDiomInputDebounceTime DebounceTime) = 0; | Sets the current pin debounce time. This is the time an input is read over to remove spurious signals. |

## OrbitModuleDIOM (Digital IO Module)

Inheritance Chain:

OrbitModuleDIOMOrbitModuleDIOMBaseOrbitModule

DIOM pins can be configured as inputs or outputs. OrbitModuleDIOM inherits many DIOM generic methods from OrbitModuleDIOMBase.

OrbitModuleDIOM Methods

| virtual void setPinConfig(int PinNumber, eDiomIoPinConfig PinConfig); | Sets the pin at PinNumber to be an input or an output pin. |
| --- | --- |

## OrbitModuleDIOM2 (Digital IO Module 2)

Inheritance Chain:

OrbitModuleDIOM2OrbitModuleDIOMBaseOrbitModule

DIOM2 modules have fixed inputs and outputs. OrbitModuleDIOM2 inherits many DIOM generic methods from OrbitModuleDIOMBase.

DIOM2 Input pins and Output pins can be configured as active high or low using bit masks. Output pins can be configured as NPN, PNP or TTL.

The module also has properties to configure the module to operate as a dynamic master. See Buffered – Sample Mode with External Master Dynamic Mode Reference and Dynamic Mode Example 2).

OrbitModuleDIOM2 Methods

| uint16_t InPolarityMask(); | Returns the current input polarity bit mask. This returns if the pins are active high or active low. |
| --- | --- |
| void setInPolarityMask(uint16_t Mask); | Sets the input pin polarity bit mask. This sets the pins to be either active high or active low. |
| uint16_t OutPolarityMask(); | Returns the current input polarity bit mask. This returns if the pins are active high or active low. |
| void setOutPolarityMask(uint16_t Mask); | Sets the output pin polarity bit mask. This sets the pins to be either active high or active low. |
| eDIOOutputMode OutputMode(); | Returns the output pin mode (PNP, NPN or TTL). |
| void setOutputMode(eDIOOutputMode Mode); | Sets the output pin mode to one of NPN, TTL or PNP. |

## OrbitModuleDIM (Digimatic Input Module)

Inheritance Chain:

OrbitModuleDIMOrbitModule

Digimatic input modules operate in three read modes, continuous, “on switch” or “on sample”. In “on switch” mode, measurements are initiated by a switch on the Digimatic device. In “on sample” mode, measurements are initiated by a sample signal sent from your application using the SampleReading()  method.

In modes other than continuous the reading can only be retrieved once. A second read will raise an Orbit Exception (reading holdoff). The NewReadingReady) method reads the module status and returns true if a new reading is ready.

The units of measurement on some Digimatic devices are configurable. The library must be made aware that the units have changed to update the OrbitModuleDIM properties using the UpdateInfo() method. It is up to your application to decide when to update this info. It is not updated on every reading due to the time required.

| bool NewReadingReady(); | Returns true if a new reading is ready. Useful when using sample on switch. |
| --- | --- |
| eDimReadMode ReadMode(); | Returns the currently configured read mode. |
| void setReadMode(eDimReadMode mode); | Sets the read mode, continuous, on switch or on sample. |
| void UpdateInfo(); | Updates the units of measurement and other reading parameters. Call this after changing the reading mode on the Digimatic device or after powering on the device. |
| void SampleReading(); | Sends a signal to the Digimatic device to instruct it to take a new reading. |

## OrbitModuleDP (Digital Probes)

Inheritance Chain:

OrbitModuleDPOrbitModule

Digital probes contain properties to set the resolution (the number of bits transmitted over the Orbit Network) and averaging (the number of sensor readings the Orbit reading is averaged over). For example 14 bits and 16 reads.

As older modules may not support resolution and averaging, and to enable us to add more values in future properties are provided to list and test the available values of these settings.

OrbitModuleDP Methods

| virtual vector<eAveraging> AvailableAveragings(); | Returns a vector of averaging values available. |
| --- | --- |
| virtual vector<eResolution> AvailableResolutions(); | Returns a vector of resolutions available |
| eAveraging Averaging(); | Returns the currently configured Averaging value. |
| bool isAverageAvailable(eAveraging Averaging); | Returns true if the averaging value is supported by the module without changing the setting. |
| bool isResolutionAvailable(eResolution Resolution); | Returns true if the resolution is supported by the module without changing the setting. |
| void setAveraging(eAveraging Averaging); | Sets how many readings the value returned by a read are averaged over. |
| eResolution Resolution(); | Gets the current resolution of reads. |
| void setResolution(eResolution Resolution); | Sets the resolution of reads. |

## OrbitModuleEIM (Encoder Input Module)

Inheritance Chain:

OrbitModuleEIMOrbitModule

Encoder input modules have properties to configure the electrical connection to the encoder and the action on passing the reference mark. Encoders do not have units of measure. A ReadingInUnits() call will return the counts value.

The setQuadratureMode() method both configures the reading mode (reading counts per edge) along with other read options.

QuadX1 – 4 edge changes per count.

QuadX2 – 2 edge changes per count.

QuadX4 – 4 edge changes per count.

CountAB – One line indicates up on edge changes, the other down.

CountDir – One line indicates direction (increment/decrement) the other counts.

The setRefAction() method configures the action to take on passing the reference mark. Use the setPresetInCounts() method to set the preset to use if the refaction is ContinuousPreset or Preset.

None – No action.

ContinuousReset – Counts are reset to zero on every pass of the ref mark.

ContinuousPreset – Presets the counts value to the preset value on every ref mark pass.

Reset – Zeros the counts value on the first pass of the reference mark only.

Preset – Presets the module to the preset value on the first pass of the ref mark only.

The setPresetInCounts() method has different effects depending on the setting for the reference mark action.

None – Immediately presets the counts value.

ContinuousReset – No action.

ContinuousPreset – Sets the preset to use on passing the ref mark.

Reset – No action.

Preset – Sets the preset to use on passing the ref mark.

This example configures an Encoder module to expect a differential electric connection, change by one count per edge and preset the module to 100 on every pass of the reference mark.

// Cast the module at index 0 to an OrbitModuleEIM pointer assuming it is an EIM

OrbitModuleEIM * eim = dynamic_cast<OrbitModuleEIM *>(network->GetModule(0));

// Configure the EIM

eim->setEimInterfaceType(eEncoderInterfaceType_Differential);

eim->setQuadratureMode(eQuadratureMode_quadX4);

eim->setRefAction(eRefAction_ContinuousPreset);

eim->setPresetInCounts(100);

The module also has properties to configure the module to operate as a dynamic master. See Buffered – Sample Mode with External Master Dynamic Mode Reference and Dynamic Mode Example 2).

| int PresetInCounts(); | Returns the last preset value configured. |
| --- | --- |
| void setPresetInCounts(int preset); | Sets the preset value. This has different effects depending on the RefAction. |
| eEncoderInterfaceType EimInterfaceType(); | Returns the current configuration for the electrical connection to the encoder, single ended or differential. |
| void setEimInterfaceType(eEncoderInterfaceType interfaceType); | Sets the current configuration for the electrical connection to the encoder, single ended or differential. |
| eQuadratureMode QuadratureMode(); | Returns the EIMs configuration for the quadrature mode (CountX1, CountX2, CountX4, CountAB or CountDir). |
| void setQuadratureMode(eQuadratureMode quadMode); | Sets the quadrature mode (CountX1, CountX2, CountX4, CountAB or CountDir). |
| eRefAction RefAction(); | Returns the configuration for the action to take on passing the encoders reference mark (None, Reset,Preset, ContinuousPreset, ContinuousReset). |
| void setRefAction(eRefAction refAction); | Sets the action to take on passing the encoders reference mark (None, Reset,Preset, ContinuousPreset, ContinuousReset). |

## OrbitModuleLE (Linear Encoder)

Inheritance Chain:

OrbitModuleLEOrbitModule

A linear encoders position reading is reset to zero on power down. To restore a linear encoders position information, it must be passed through its reference mark.

The FindRefMark_Units() and FindRefMark_counts() methods of the OrbitModuleLE start a thread that queries the linear encoder module until the reference mark is found or the find reference mark operation is cancelled.

The class also has methods to set the reading direction (increment or decrement reading on retract) and to immediately preset the module reading to value, either in counts or units of measure.

An example implementation can be found in the OrbitLinuxLibraryTest application (referencemarkwdiget.cpp).

An example of learning an LE ref mark:

// Cast the module at index 0 to an OrbitModuleLE pointer assuming it's an LE

OrbitModuleLE * le = dynamic_cast<OrbitModuleLE *>(network->GetModule(0));

// Begin the learn reference mark operation

le->FindRefMark_Counts(eReferenceMarkMode_Learn, 0);

// Loop until found

while (le->RefmarkInProgress())

{

sleep(1);

}

// Print out the counts value at the reference mark

if (le->RefmarkFound())

cout << "Ref Mark Datum Counts: " << le->ReadInCountsAtRefmark();

else

cout << "Ref Mark Not Found:”;

cout << endl;

An example of recalling an LE ref mark with a counts reading of 23970:

// Cast the module at index 0 to an OrbitModuleLE pointer assuming it's an LE

OrbitModuleLE * le = dynamic_cast<OrbitModuleLE *>(network->GetModule(0));

// Begin the recall reference mark operation

le->FindRefMark_Counts(eReferenceMarkMode_Recall, 23970);

// Loop until found

while (le->RefmarkInProgress())

{

sleep(1);

}

OrbitModuleLE Methods

| int PresetInCounts(); | Returns the last value that the LE was preset to in Counts. |
| --- | --- |
| void setPresetInCounts(int presetValue); | Immediately presets the LE to the counts value in presetValue. |
| double PresetInUnits(); | Returns the last value that the LE was preset to in units of measure. |
| void setPresetInUnits(double presetValue); | Immediately presets the LE to the value in presetValue specified in units of measure. |
| eDirection Direction(); | Retrieves the last value set with the setDirection() method. |
| void setDirection(eDirection directionToSet); | Sets the LE reading direction value (Normal = increment on retract; Reverse = increment on extend). |
| double ReadInCountsAtRefmark(); | Returns the counts value at the ref mark. The reference mark must have been found before calling this method by using the FindRefMark_Counts() method. |
| double ReadInUnitsAtRefmark(); | Returns the units value at the ref mark. The reference mark must have been found before calling this method by using the FindRefMark_Units() method. |
| bool RefmarkInProgress(); | Returns true whilst a FindRefMark operation is in progress |
| bool RefmarkFound() | Returns true if the reference mark has been found |
| void FindRefMark_Counts(eReferenceMarkMode mode, int valueToSetAtRefMark); | Starts a find reference mark operation. The mode parameter specifies learn or recall. valueToSetAtRefMark is the counts value at the reference mark to be used in a recall operation. valueToSetAtRefMark is ignored in a recall operation. |
| void FindRefMark_Units(eReferenceMarkMode mode, double valueToSetAtRefMark); | Starts a find reference mark operation. The mode parameter specifies learn or recall. valueToSetAtRefMark is the value in units of measure at the reference mark to be used in a recall operation. valueToSetAtRefMark is ignored in a recall operation. |
| void StopRefMark(); | Cancels a find reference mark operation. |

## OrbitModuleLT (Laser Triangulation)

Inheritance Chain:

OrbitModuleLTOrbitModuleAIMOrbitModuleDPOrbitModule

Refer to the Laser Sensor manual (503518) for full laser product instructions.

Laser triangulation modules are non-contact sensors. Orbit supports three kinds, basic LT and LTA lasers and more advanced LTH lasers. This class is for the basic LT modules. DP properties are inherited by this module type (see the section OrbitModuleDP (Digital Probes) for the reference).

LT, LTA, and LTH modules default to beam on at power up. Properties are provided to turn off the beam. The laser must be turned on to return readings.

OrbitModuleLT Methods

| bool BeamOn(); | This returns the last value set for the BeamOn value. Note: This does not query the module for the actual beam state. |
| --- | --- |
| void setBeamOn(bool BeamSetting); | Changes the beam state (true = on, false = off). |

## OrbitModuleLTA (Laser Triangulation)

Inheritance Chain:

OrbitModuleLTAOrbitModuleLTOrbitModuleAIMOrbitModuleDPOrbitModule

Refer to the Laser Sensor manual (503518) for full laser product instructions.

Laser triangulation modules are non-contact sensors. Orbit supports three kinds, basic LT and LTA lasers and more advanced LTH lasers. This class is for the more basic LTA modules. LT and DP properties are inherited by this module type (see the sections OrbitModuleLT (Laser Triangulation) and OrbitModuleDP (Digital Probes) for the reference).

LT, LTA, and LTH modules default to beam on at power up. Properties are provided to turn off the beam. The laser must be turned on to return readings.

| vector<eMeasuringRate> AvailableMeasuringRates(); | A list of the measuring rates that the module is capable of. |
| --- | --- |
| bool isMeasuringRateAvailable(eMeasuringRate MeasuringRate) | Returns true if the module is capable of the measuring rate passed in. |
| eMeasuringRate MeasuringRate(); | Returns the current measuring rate (after the rate has been set the first time). |
| void setMeasuringRate(eMeasuringRate MeasuringRate); | Changes the current measuring rate. |

## OrbitModuleLTH (Laser Triangulation High Sensitivity)

Inheritance Chain:

OrbitModuleLTHOrbitModuleLTOrbitModuleAIMOrbitModuleDPOrbitModule

Refer to the Laser Sensor manual (503518) for full laser product instructions.

Laser triangulation modules are non-contact sensors. Orbit supports three kinds, basic LT and LTA lasers and more advanced LTH lasers. This class is for the more advanced LTH modules. As the module inherits from OrbitModuleLT the beam can be controlled with setBeamOn().

Properties are provided for LTH modules to read information about the laser module, set the low pass filter and level cut time. The low pass filter improves signal stability. The level cut time will continue to output the last valid reading for the time specified when the reading goes out of range. This is useful for example when the laser passes over a slot in the surface of the item being measured.

LT, LTA, and LTH modules default to beam on at power up. Properties are provided to turn off the beam. The laser must be turned on to return readings

OrbitModuleLTH Methods

| string ModelNumber(); | Returns the laser head model number. |
| --- | --- |
| string SerialNumber() | Returns the laser head serial number. |
| string FirmwareNumber(); | Returns the laser head firmware version. |
| uint16_t LevelCutTimeMax(); | Returns the maximum value that the level cut time can be set to. |
| uint16_t LevelCutTime(); | Returns the level cut time that the laser head is set to. |
| void setLevelCutTime(uint16_t value); | Sets the level cut time to the value specified. |
| eFilter Filter(); | Returns the current low pass filter setting value. |
| void setFilter(eFilter value); | Sets the low pass filter to the value specified |

## OrbitModuleWCM (Wireless Communication Module)

A WCM module performs Bluetooth communications between an Orbit network and Solartron wireless handtools. A WCM can communicate with up to six hand tools.

The WCM must be configured with the Orbit Identity of a hand tool before communications can be established. The Orbit Identity is stored in non-volatile memory in the WCM (therefore it is not necessary to configure it on each power up). To remove a WHT from a WCM, clear its Orbit Identity and restart the WCM.

The code sample below is an example of how to configure a WCM with the Orbit identity of a handtool and read every channel on the device. It does not include adding the WCM to the network (see Adding Modules to an Orbit Network).

// Add the WHT by setting the Orbit Identity

// In your code. This should only be done once

// the WCM will store the Orbit Identity in non

// volatile memory.

// Assuming WCM is at index 0

OrbitModuleWCM *wcm = dynamic_cast<OrbitModuleWCM *>(network->GetModule(0));

// Get the first handtool from the WCM_Devices object

WCM_Device *wht = wcm->Devices()->GetDevice(0);

// Set the Orbit Identity

wht->setOrbitIdentity("114AJ16701");

// Restart the WCM to apply changes

// This may take a few seconds

wcm->Restart();

// Read the hand tool

for(int reading=0; reading < 60; reading++)

{

cout << "Reading " << reading << ": ";

try

{

// We need to read the device to know how many channels it has

if (wht->NumChannels() == 0)

{

// Update the number of channels etc.

wht->UpdateDeviceFieldData();

}

OrbitChannelReadings readings = wht->ReadAllChannels();

for (int channel = 0; channel < readings.ReadingCount(); channel++)

{

// Output the channel readings

cout << readings[channel].Reading;

// Print comma between channels or EOL

if (channel < (readings.ReadingCount() - 1))

cout << ", ";

else

cout << endl;

}

}

catch (runtime_error& error)

{

cout << error.what() << endl;

}

sleep(1);

}

OrbitModuleWCM Methods

| WCM_Devices* Devices(); |  | Returns the WCM_Devices object used to access WHT devices. |
| --- | --- | --- |
| void Restart(); |  | Restarts the WCM. The WCM must be restarted after making configuration changes, for example adding the Orbit Identity of a WHT. |

# Wireless Hand Tools

Wireless hand tools (WHTs) are Bluetooth devices. The Linux library does not allow direct communication with hand tools, all communication is via a WCM (wireless communication module) on an Orbit Network. WHT and WHT-M hand tool types are supported by the Linux library.

For example, to get the first WHT from a WCM that is the first module on the network access:

// Assuming WCM is at index 0

OrbitModuleWCM *wcm = dynamic_cast<OrbitModuleWCM *>(network->GetModule(0));

// Get the first handtool from the WCM_Devices object

WCM_Device *wht = wcm->Devices()->GetDevice(0);

The WCM_Devices object of a WCM manages each WCM device. A WCM currently always has six devices.

WHTs are not read using the same mechanism as Orbit Modules. The WCM_Device class has a number of polymorphic Read() and ReadAllChannels() methods. The WCM_Device object for a WHT must be retrieved from the WCM that it is configured with. See OrbitModuleWCM for an example of how to configure a WHT with a WCM and read it.

WHT device channels can be tagged or untagged. Readings are tagged by configuring one of the handtool buttons as a tag button. Reads can be either of two types, 7dp shifted integers (the reading*10^7) or floating point. The library will convert the readings from either type to both floating point and integer representations. Readings can be Normal, Minimum, Maximum or Difference.

WCM_Devices Methods

| WCM_Device* GetDevice(int index); | Gets the WCM device at the index. |
| --- | --- |
| int DeviceCount(); | Returns the number of devices that the WCM supports (currently always 6). |

The WCM Device object contains the basic properties of both WHT and WHT-M handhelds and the read methods. Readings are returned in an OrbitChannelReadings struct. The WCM will not be aware of the WHT properties until the WHT connects with the WCM.

WCM Device Methods

| string OrbitIdentity(); | Returns the WHT Orbit Identity (The identity configured for the WHT on the WCM). |
| --- | --- |
| void setOrbitIdentity(string identity); | Sets the Orbit Identity for the WHT. After setting the Orbit Identity, the WCM must be restarted. |
| eWCM_DeviceType DeviceType(); | Returns the WHT device type (None, WHT, WHT-M). |
| string DeviceName(); | Returns the user set device name. |
| void setDeviceName(string name); | Allows the user to set a device name for the WHT device. |
| void UpdateDeviceFieldData(); | Updates all the device field data, including the number of channels, device type, channel stroke etc.) |
| eWCM_DeviceStatus ConnectionStatus(); | Returns the connection status (Unknown, Not Connected, Connected, Connecting, Error). |
| int BatteryStatus(); | Returns the battery status of the WHT as a percentage. |
| int NumChannels(); | Returns the number of channels on the WHT |
| OrbitChannelData GetChannelData(int ChannelIndex); | Returns an OrbitChannelData struct containing the Orbit Identity and Stroke of each channel. |
| OrbitChannelReadings Read(unsigned int ChannelMask, eRead3_ControlCode ControlCode, eRead3_UOM UOM); | Reads the channels specified by the bit mask using the read mode specified by ControlCode (Normal/Tagged/Int/Float) in the UOM specified (inches/mm/default). |
| OrbitChannelReadings Read(unsigned int ChannelMask, eRead3_ControlCode ControlCode); | Reads the channels specified by the bit mask using the read mode specified by ControlCode (Normal/Tagged/Int/Float) in the devices default UOM. |
| OrbitChannelReadings Read(eChannelMask ChannelMask, eRead3_ControlCode ControlCode); | Reads the channel(s) specified by the ChannelMask using the read mode specified by ControlCode (Normal/Tagged/Int/Float) in the devices default UOM. |
| OrbitChannelReadings Read(eChannelMask ChannelMask); | Performs a normal 7dp int read of the channel(s) specified by ChannelMask in the devices default UOM; |
| OrbitChannelReadings Read(unsigned int ChannelMask); | Performs a normal 7dp int read of the channel in the devices default UOM; |
| OrbitChannelReadings ReadAllChannels(eRead3_ControlCode ControlCode, eRead3_UOM UOM); | Reads all channels using the read mode specified by ControlCode (Normal/Tagged/Int/Float) in the UOM specified (inches/mm/default). |
| OrbitChannelReadings ReadAllChannels(eRead3_ControlCode ControlCode); | Reads all channels using the read mode specified by ControlCode (Normal/Tagged/Int/Float) in the devices default UOM. |
| OrbitChannelReadings ReadAllChannels(); | Performs a 7dp integer read of all channels in the devices default UOM |
| WCM_DeviceSettingsBase* Settings(); | Returns an object to configure a WHT’s settings. The class returned will depend on the WHT Type. It will either be a WCM_DeviceSettings_WHT or WCM_DeviceSettings_WHTM. |

OrbitChannelData Struct

| string ChannelIdentity; | The Orbit Identity of the channel. |
| --- | --- |
| double Stroke; | The channel stroke. |

OrbitChannelReadings Struct

| int BatteryStatus; | The battery percentage remaining. |
| --- | --- |
| int ChannelIndex; | The channel index. Note: The Index is 1 based. |
| eWCM_DeviceLimitState LimitStatus; | The limit status (OK, invalid, upper, lower, warnUpper, warnLower). |
| eWCM_DeviceOutputMode OutputMode; | The output mode (Normal, Min, Max, Diff, TaggedNormal, TaggedMin, TaggedMax TaggedDiff) |
| eWCM_DevicePZAMode PZAMode; | The reading mode (preset, zero or abolute). |
| double Reading; | The reading expressed as a double floating-point number. If the read mode is an Integer mode, this will have been converted from the integer read from the WHT. |
| int ReadingInt; | The reading expressed as a scaled 7 DP integer (10^7 * reading). If the read mode is float, this will have been converted from the float received from the WHT. |
| eOrbitErrors Status; | The reading status (NoError, UnderRange, OverRange etc.) |
| int TagNumber; | The tag number, only populated when using tagged readings. |

## Wireless Handtool Configuration

Wireless Handtools (both WHT and WHT-M type) are configured using ASCII commands. This library includes classes to automate the transmission of the commands.

WHT and WHT-M hand tool types have different capabilities. Calling the WCM_Device::Settings() method will return an object that can be cast from the base WCM_DeviceSettingsBase type to a settings object  of the correct type for the hand tool (WCM_DeviceSettings_WHT or WCM_DeviceSettings_WHTM).

The library automatically sets the hand tool into configuration mode when sending commands, however if multiple settings are to be sent/retrieved at the same time it will be faster to manually switch into and out of configuration mode using the setConfigMode() method of the settings classes.

After using setConfigMode(true) to set the handtool into config mode, your application should poll EnteredConfigMode() until EnteredConfigMode() returns true indicating that the hand tool is connected and is in config mode. When your application has finished retrieving/setting the WHT settings, use setConfigMode(false) to resume normal WHT operation.

Refer to the Orbit Linux Library Test Application for an example implementation.

### WCM_DeviceSettingsBase

These base class methods are applicable to either hand tool type.

| bool ConfigMode(); | Returns true if the WHT is set to enter config mode. This does not indicate that the WHT is in config mode, to test that poll EnteredConfigMode(); |
| --- | --- |
| void setConfigMode(bool value); | Commands the WHT to enter or exit config mode. |
| bool EnteredConfigMode(); | Returns true once connecting and setting the hand tool into config mode has been completed. |
| string SendCommand(string CommandSetting, bool ReplyExpected); | Sends an ASCII command to a hand tool. If a reply to the command is expected, set ReplyExpected to true and the reply will be returned. If the hand tool is in settings mode the command is simply sent. If the hand tool is not in settings mode, the command will attempt to set the hand tool into settings mode, send the command and then switch back to normal mode. |

### WCM_DeviceSettings_WHT

These settings are applicable only to single channel (WHT) hand tools.

| void ABS(); | Clears any zero or preset from the reading setting the WHT to absolute readings. |
| --- | --- |
| int AutoPowerDownTime(); | Returns the time in seconds between a WHT button being pressed and the WHT auto powering down. If the value returned is zero, auto power down is disabled |
| void setAutoPowerDownTime(int value); | Sets the time in seconds between a WHT button being pressed and the WHT auto powering down. Set the value to zero to disable auto power down. |
| eWHT_BluetoothClass BluetoothClass(); | Returns the WHT Bluetooth class (transmit power) |
| void setBluetoothClass( eWHT_BluetoothClass value); | Sets the WHT Bluetooth class (transmit power). Changes will only be applied after the WHT has been power cycled. |
| bool ButtonBuzzerEnable(); | Returns true if the buzzer is enabled on button presses. |
| void setButtonBuzzerEnable(bool value); | Turns the buzzer on button presses on or off. |
| bool GetButtonFunctions( eWHT_ButtonFunction &Button1Fn, eWHT_ButtonFunction &Button2Fn); | Returns the button functions configured for the WHT buttons (Tag, Zero or Preset). |
| int DisplayedResolution(); | Returns the number of decimal places displayed on the WHT display. |
| void setDisplayedResolution(int value); | Sets the number of decimal places displayed on the WHT display. Values between 2 and 5 are valid. |
| eWHT_DisplayLargeSource DisplayLargeSource(); | Returns if the WHT is configured to display the live or computed readings whilst the WHT display is in large mode. |
| void setDisplayLargeSource( eWHT_DisplayLargeSource value); | Sets the WHT display the live or computed readings whilst the WHT display is in large mode. |
| eWHT_DisplayLayout DisplayLayout(); | Returns the display layout, standard (detailed) or large. |
| setDisplayLayout(eWHT_DisplayLayout value); | Sets the display layout to standard (detailed) or large. |
| eWHT_DisplayOrientation DisplayOrientation(); | Returns the WHT display orientation setting (0, 90, 180, 270 or auto). |
| void setDisplayOrientation( eWHT_DisplayOrientation value); | Sets the WHT display orientation (0, 90, 180, 270 or auto). |
| eWHT_OperationMode OperationMode(); | Returns the WHT operation mode (Normal, Min, Max, Difference, TaggedNormal, TaggedMin, TaggedMax, TaggedDifference). |
| void setOperationMode(eWHT_OperationMode value); | Sets the WHT operation mode (Normal, Min, Max, Difference, TaggedNormal, TaggedMin, TaggedMax, TaggedDifference). |
| eWHT_PowerOffButtonOptions PowerOffButtonOptions(); | Returns which buttons can power off the WHT (None, Button1, Button2, Either or Both). |
| void setPowerOffButtonOptions( eWHT_PowerOffButtonOptions value); | Configures which buttons can power off the WHT(None, Button1, Button2, Either or Both). |
| void Preset(); | Presets the WHT to PresetValue. PresetValue can be set using setPresetValue(). |
| double PresetValue(); | Returns the value that will be used to preset the WHT when Preset() is called or a preset button is pressed. |
| bool LimitBuzzerEnable(); | Returns if the limit buzzer is enabled. |
| void setLimitBuzzerEnable(bool value); | Turns the limit buzzer on or off. |
| eWHT_LimitBuzzerRate LimitBuzzerRate(); | Returns the limit buzzer rate (three fast or one slow). |
| void setLimitBuzzerRate( eWHT_LimitBuzzerRate value); | Sets the limit buzzer rate (three fast or one slow). |
| eWHT_LimitLED LimitLED(); | Returns the LED settings for if a limit is breached (None, green red or both). |
| void setLimitLED(eWHT_LimitLED value); | Configures the LED settings for if a limit is breached (None, green red or both). |
| WHT_Limits* Limits(); | Returns an object to configure the WHT limits. |
| void ResetFactorySettings(); | Resets the WHT to factory default settings. After calling this method the WHT must be power cycled before changes will take effect. |
| void ResetMinMax(); | Resets the min and max values in MinMax modes. |
| void SaveZero(); | Saves the WHT zero and preset values so that they are restored if the WHT is power cycled. |
| void setPresetValue(double value); | Sets the PresetValue to apply to readings using Preset() or a preset button. Does not apply the PresetValue to readings immediately. |
| void SetAndApplyPreset(double PresetToApply); | First sets the PresetValue then immediately applies the preset to readings. |
| void SetButtonFunctions(eWHT_ButtonFunction Button1Fn, eWHT_ButtonFunction Button2Fn); | Configures the WHT buttons. Both buttons can be set to Tag, Zero or Preset. |
| void Zero(); | Zeros the WHT reading. |

WHT Limits are configured using the WHT_Limits object returned from WHT_Device::Limits(); The actual upper and lower limits are controlled by a WHT_Limit object returned from a WHT_Limits::Upper() or WHT_Limits::Lower() call.

WHT_Limits

| bool Enabled(); | Returns true if the WHT limits are enabled. |
| --- | --- |
| void setEnabled(bool value); | Turns the WHT limits on or off. |
| WHT_Limit* Upper(); | Returns a pointer to the upper limit object. |
| WHT_Limit* Lower(); | Returns a pointer to the lower limit object. |

WHT_Limit

| double Value(); | Returns the limit threshold (value to check against). |
| --- | --- |
| void setValue(double value); | Sets the limit threshold (value to check against). |

### WCM_DeviceSettings_WHTM

These settings are applicable only to multichannel (WHTM) hand tools. Channel numbers are 1 based.

| eWHT_BluetoothClass BluetoothClass(); | Returns the hand tool Bluetooth class (transmit power). This is not settable via the WCM. |
| --- | --- |
| eWHT_M_ButtonFunction Button1Function(); | Returns the function configured for button 1 (None, tag, power down). |
| void setButton1Function( eWHT_M_ButtonFunction value); | Configures WHTM button 1 functionality (None, tag, power down). |
| eWHT_M_ButtonFunction Button2Function(); | Returns the function configured for button 2 (None, tag, power down). |
| void setButton2Function( eWHT_M_ButtonFunction value); | Configures WHTM button 2 functionality (None, tag, power down). |
| eWHT_M_BuzzerFunction BuzzerFunction(); | Returns the WHTM buzzer function (None, or TagTaken) |
| void setBuzzerFunction( eWHT_M_BuzzerFunction value); | Configures the WHTM buzzer function (None, or TagTaken |
| eWHT_M_DisplayMode DisplayMode(); | Returns if the WHTM is set to display numbers or charts. |
| void setDisplayMode( eWHT_M_DisplayMode value); | Configures the WHTM to display numbers or charts. |
| eWHT_DisplayOrientation DisplayOrientation(); | Returns the WHTM display orientation setting (0, 90, 180, 270 or auto). |
| void setDisplayOrientation( eWHT_DisplayOrientation value); | Sets the WHTM display orientation (0, 90, 180, 270 or auto). |
| eWHT_M_LEDFunction LED1Function(); | Returns the functionality configured for LED 1 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| void setLED1Function( eWHT_M_LEDFunction value); | Sets the functionality for LED 1 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| eWHT_M_LEDFunction LED2Function(); | Returns the functionality configured for LED 2 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| void setLED2Function( eWHT_M_LEDFunction value); | Sets the functionality for LED 2 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| eWHT_M_LEDFunction LED3Function(); | Returns the functionality configured for LED 3 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| void setLED3Function( eWHT_M_LEDFunction value); | Sets the functionality for LED 3 (Off, Indicate Streaming, Indicate Range Error, Indicate No Range Error, Indicate Battery Low). |
| int OffTime(); | Returns the time in seconds between the last WHTM button press and the WHTM auto powering down. A value of zero indicates auto power down is disabled. |
| void setOffTime(int value); | Sets the time in seconds between the last WHTM button press and the WHTM auto powering down. A value of zero disables auto power down. |
| void ResetFactorySettings(); | Resets the WHTM to factory default settings. After calling this method the WHTM must be power cycled to apply the changes. |
| int GetDisplayedResolution(int ChannelNumber); | Returns the display resolution for a channel (number of decimal places). |
| void SetDisplayedResolution(int ChannelNumber, int DisplayedResolution); | Sets the display resolution for a channel (number of decimal places). |

### WHT Settings Example

This example sets the WHT into configuration mode and zeros the reading.

// Cast the module at index 0 to an OrbitModuleWCM pointer assuming it is a WCM

OrbitModuleWCM *wcm = static_cast<OrbitModuleWCM *>(network->GetModule(0));

// Get the handtool device object from the WCM

WCM_Device *device = wcm->Devices()->GetDevice(0);

// Make sure we have a WHT and not a WHTM

if (device->DeviceType() == eWCM_DeviceType_WHT)

{

// Get the settings object

WCM_DeviceSettings_WHT *settings

= static_cast<WCM_DeviceSettings_WHT *>(device->Settings());

// Enter Config Mode

settings->setConfigMode(true);

while (!settings->EnteredConfigMode())

{

sleep(1);

}

// Zero the reading

settings->Zero();

// Exit Config Mode

settings->setConfigMode(false);

}

else

cout << "Incorrect hand tool type" << endl;

# Revision History

| REVISION | DATE | COMMENTS |
| --- | --- | --- |
| 1 |  | Initial Issue |